APPJ Startup/Shutdown Procedure

Mesbah Lab 2021

Last Updated: February 14, 2021

WARNING: The APPJ setup involves high voltage equipment! Use caution during operation, and DO NOT touch/reach into any part of the setup within the PVC cage!!

STARTUP:

- Open Gas Flow (see Figure 1 for Helium tank setup):

  - Ensure regulator (large red knob in center) is decreased fully.

  - Open tank valve on the top of the He gas tank. The gauge on the right should indicate some pressure.

  - Increase the regulator (turn clockwise) until the left gauge shows an appropriate amount of pressure (enough to get up to 3-5 slm).

  - Open the line valve on the left (turn parallel to gas line)

Line Valve

Regulator

Tank Valve

Line Valve

Regulator

Tank Valve

Figure 1: Helium Tank Setup

Figure 1: Helium Tank Setup

- Turn on Amplifier for Frequency Generator (See Figure 2 for Amplifier startup):

  - Flip the power switch to turn on power.

  - Pull gray knob to unlock high voltage lock.

  - Push white button to turn on high voltage. HV ON indicator light should light up green.

CAUTION: Once this is powered on, do NOT touch anything in the setup!!! Risk of high voltage delivered to body!

Figure 2: Amplifier Power Switches

Power Switch

High Voltage Lock

HV Local ON Button

Figure 2: Amplifier Power Switches

Power Switch

High Voltage Lock

HV Local ON Button

- Connect Computer to setup:

  - Locate the USB Hub.

  - Connect to computer.

- Startup APPJ by EITHER:

  - Running Python script: APPJstartup.py provided in APPJ-MacOS-Communication (ensure required dependencies/libraries for Python are installed with pip)

  - Connecting to the Arduino directly using the following commands in the Terminal:

    - This line starts/connects the Arduino to the computer and reads the status of the connection:
$ stty -f /dev/cu.usbmodem1434401 raw 38400 -hupcl & cat /dev/cu.usbmodem1434401
Ensure the `usbmodem1434401` device number is consistent with what is read by your computer. If you have the Arduino app installed on your computer, this can be checked by going to Tools > Port, and looking at the Arduino device number.

    - Then, the following commands can be used to ignite the plasma:

      - This command sets the duty to 100:
$echo "p,100" > /dev/cu.usbmodem1434401

      - This command sets the power to 2:
$echo "w,2" > /dev/cu.usbmodem1434401

      - This command sets the flow rate to 1.5:
$echo "q,1.5" > /dev/cu.usbmodem1434401

    - Once started, the same commands can be used to adjust those parameters (duty, power, flow rate). Just change the numerical value in the quotes.

SHUTDOWN:

- Push the reset button on the Arduino to reset all parameters.

- Turn off Amplifier (see Figure 2):

  - Flip the power switch to the off position.

  - Push the gray knob to lock.

- Turn off Gas (see Figure 1):

  - Close tank valve on the top of the tank.

  - Go to setup. On the manual flow controller (see Figure 3), flip the switch to ‘Purge’ to purge the remaining gas in setup.

  - Decrease the regulator fully (turn counterclockwise).

  - Close line valve (turn perpendicular to gas line).

Figure 3: Manual Flow Controller

Figure 3: Manual Flow Controller